# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of Equipment for pilot composting facility at PE Komunalec – Bitola 	p 1 /…

Publication reference: SYM-BIT-2

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Lot 1: In-vessel composter with loading unit (shredding & bin lifting)

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | In-vessel aerobic composter (1 pcs). Capacity: min 12 m3 per month / min 6 ton per month with density of a 0.5 kg/litter. Duration of composing cycle: <= 30 days Composter should be able to process different type of organic waste: - food waste - food processing waste - green waste - agricultural waste The composter should be equipped with loading unit with shredder and bin lifting system for at least 120 l organic waste bins |  |  |  |

Lot 2: Vehicle for collection of organic waste (upgrade container capacity 12 m3)

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Vehicle for collection of organic waste (1 pcs) Truck specifications: • Inter-axial distance maximum 3700 mm; • Gross mass max 16000 kg; • Engine: 6 cylindrical, diesel with direct injection of fuel • Engine volume max 6000 cc, • Power maximum 185 kW; • Emission min Euro 5; • Drive 4x2; • Parabolic springs on front and rear axles with shock absorbers; • Transmission: synchronous manual with 6 speeds; • Braking system: front and rear discs, ABS + ASR engine, service and parking brake; • Driver's cab short with 1 + 2 seats; • Double passenger seat; • Rear drive axle with mechanical differential lock. • Speed ​​limiter at max 90 km / h; • Digital tachograph; • Exhaust mounted vertically upwards • Air compressor max 250 cc; • Audible signalling for backward movement; • Spare wheel with carrier; • Hydraulic steering system; • Rotary lights; • Air conditioning in the driver's cabin. Upgrade container specification: • Volume of the container for collection of waste min 12 m3; • Cover of the container made of sheet metal with a thickness of 3 mm • Installed rubber profile for sealing the door of the waste container; • Volume of the rear waste container (basket) 1.5-2.5 m3; • Compaction of waste 5:1 • System of plate under pressure for loading and unloading of waste with overload protection; • System installed in the background of container for lifting and discharging waste bins with capacity of 1,1 m3, 240l and 120l (plastic containers) in accordance with EN 8401,2,3 • Automatic and manual control of the working cycle, with the possibility of changing to manual control at a certain point; • A device for automatically increasing and limiting the engine RPM in order to achieve the required pressure for safe operation of the hydraulic system of the upgrade, • The waste treatment should be carried out with a combined electric and hydraulic system, • Two folding ladders on the rear for transport of the workers; • Two handles on the rear for secure transport of the workers; • Two working lights on the rear of the vehicle, one lamp for the vehicle waste container, and one lamp for the work surface; • Safety valve on the hydraulic cylinder for raising the door • Rotary safety lamps • The material for making the collecting chest should be of high-quality steel with a thickness of 8mm; • Holders and parts for loading on the steel structures should be made of high-quality steel • High quality protection of all components, electro-sanded, coloured with two-component paint in white. |  |  |  |

Lot 3: Organic waste bins (capacity: 120l, 240l, 1.1 m3)

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | PVC bin with a volume of 120 litres (2000 pcs). - Purpose: for the collection of organic waste - Bin and lid material: PVC resistant on high- and low-temperatures, UV rays and infrared rays, resistant to chemical and biological impacts; - The inner surface, a smooth that restricts stuck of the waste; - The lid flat with min. 1 handle for manual opening; - To have specialized ventilation openings; - Thickness of material min 3mm, in hardened places min 5mm; - Volume min 120 litres; - Colour brown; - Nominal load of a maximum of 60 kg; - Standard version with two fixed wheels connected to a metal stainless steel shaft; - Wheels made with solid rubber tire and a plastic base without bearings, with axes; Dimensions and performances to be according to MKS EN840-1:2013 or equivalent. |  |  |  |
| 2 | PVC bin with a volume of 240 litres (200 pcs). - Purpose: for the collection of organic waste - Bin and lid material: PVC resistant on high- and low-temperatures, UV rays and infrared rays, resistant to chemical and biological impacts; - The inner surface, a smooth that restricts stuck of the waste; - The lid flat with min. 1 handle for manual opening; - To have specialized ventilation openings; - Thickness of material min 3mm, in hardened places min 5mm; - Volume min 240 litres; - Colour brown; - Nominal load of a maximum of 120 kg; - Standard version with two fixed wheels connected to a metal stainless steel shaft; - Wheels made with solid rubber tire and a plastic base without bearings, with axes; Dimensions and performances to be according to MKS EN840-1:2013 or equivalent. |  |  |  |
| 3 | PVC container with a volume of 1,1 m3 (35 pcs). - Purpose: for the collection of organic waste - Container and lid material: PVC resistant on high- and low-temperatures, UV rays and infrared rays, resistant to chemical and biological impacts; - The inner surface, a smooth that restricts stuck of the waste; - The lid flat with min. 1 handle for manual opening; - To have specialized ventilation openings; - Thickness of material min 3mm, in hardened places min 5mm; - Volume min 1,1 m3; - Colour brown; - Nominal load of a maximum of 550 kg; - Standard version with four wheels, two of them secured with brakes; - Wheels made with of solid rubber tire and a plastic base without bearings, with axes; Dimensions and performances to be according to MKS EN840-2:2013 or equivalent. |  |  |  |